RTL_EXERCISE_1 BOUND FLASHER

Author; Trần Anh Kiệt. Date; 5/3/2024. Version; 1.1.

Contents

1. Interface	2

2. Functional implementation.	3

3. Internal implementation.	5

3.1. Overall.	5

3.2. State Machine	6

4. History	7

# 1. Interface

Bounce_flash clock 16 lamps flick reset

Figure 1: the figure of Bound Flasher System

| Signal | Width | In/Out | Description |
| --- | --- | --- | --- |
| Clock | 1 | In | The clock signal that drives the state changes and timing within the module. Typically connected to the system clock. |
| Reset | 1 | In | Active-low reset signal. When asserted (0), it initializes the module's state to INIT and resets the lamps and lamp_counter. |
| Flick | 1 | In | Input signal used to trigger specific lamp behavior. When asserted (1), it activates flicker logic affecting the current lamp pattern. |
| lamps | 16 | Out | Output signal representing the state of 16 lamps. Each bit corresponds to a lamp, where 1 indicates on and 0 indicates off. |

Table 1: Description of signals in Bound Flasher

# 2. Functional implementation.

- Implement a 16-bits LEDs system

- System’s Operation base on three input signal

  - Reset

  - Clock

  - Flick

- The system specification

- Clock signal is provided for system inspire of function status. The function operate state’s transition at positive edge of the clock signal.

- Reset signal:

- LOW-ACTIVE Reset = 0: System is restarted to Initial State.

- HIGH-ACTIVE Reset = 1: System is started with initial state.

- Flick signal: special input for controlling state transfer.

- At the initial state, all lamps are OFF. If flick signal is ACTIVE, the flasher start operating:

- The lamps are turned ON gradually from LEDs [0] to LEDs [15].

- The LEDSs are turned OFF gradually from LEDs [15] to LEDs [5].

- The LEDSs are turned ON gradually from LEDs [5] to LEDs [10].

- The LEDSs are turned OFF gradually from LEDs [10] to LEDs [0].

- The LEDSs are turned ON gradually from LEDs [0] to LEDs [5].

- Finally, the LEDs s are turned OFF gradually from LEDSs [5] to LEDSs [0], return to initial state.

- Additional condition: At each kickback point (LEDs [5] and LEDs [0]), if flick signal is ACTIVE, the LEDs will go back and repeat that STATE. For simple, kickback point is considered only when the LEDs s are turned OFF gradually, except final state.

- Some insulations:

- When flick = 0 at kickback points

- When flick = 1 at kickback points (lamp[5])

# 3. Internal implementation.

## 3.1. Overall.

Figure 3.1: Block diagram of Bound Flasher

Signal; Width; In/Out; Description. new_buf_req; 1; In; Signal to request a new buffer.. ref_buf_numbr; 2; In; Reference buffer number input signal.. clk; 1; In; The clock signal that drives the state changes and timing within the module.. rst_n; 1; In; Active-low reset signal. When asserted (0), it initializes the module’s state.. next_access_time; 8; Out; Next access time output from access control logic combination.. access_time; 8; Out; Access time output from D-FF component after processing next_access_time.. buf_temp_replc; 2; Out; Buffer template replacement output after comparison in control logic combination.. buf_num_replc; 2; Out; Buffer number replacement output after comparison in control logic combination..

Table 3.1: Block diagram of Bound Flasher Description

## 3.2. State Machine

Figure 3.2: State Machine of Bound Flasher

| Variable Name | Description |
| --- | --- |
| current_state | Holds the current state of the state machine. |
| next_state | Intended to hold the next state (not used in the provided code). |
| flick_active | A flag to indicate if the flick signal has been activated. |
| lamp_counter | A counter to keep track of the current lamp pattern. |

Table 3.2: variable name of State machine

| State Name | Description |
| --- | --- |
| INIT | Initial state where all lamps are turned off. The system transitions to TURN_ON_TO_5 next. |
| TURN_ON_TO_5 | Lamps are sequentially turned on from 0 to 5. After reaching 5, transitions to TURN_OFF_TO_0. |
| TURN_OFF_TO_0 | Lamps are sequentially turned off from 5 to 0. After all are off, transitions to TURN_ON_TO_10. |
| TURN_ON_TO_10 | Lamps are sequentially turned on from 0 to 10. If flick is activated at lamp 5, it flicks before continuing. After reaching 10, transitions to TURN_OFF_TO_5. |
| TURN_OFF_TO_5 | Lamps are sequentially turned off from 10 to 5. After reaching 5, transitions to TURN_ON_TO_15. |
| TURN_ON_TO_15 | Lamps are sequentially turned on from 5 to 15. If flick is activated at lamp 10, it flicks before continuing. After reaching 15, transitions to TURN_OFF_TO_0_AGAIN. |
| TURN_OFF_TO_0_AGAIN | Lamps are sequentially turned off from 15 to 0. After all are off, transitions to BLINK. |
| BLINK | All lamps are turned on, then all are turned off after a brief moment. The system then resets to INIT. |

Table 3.3: state name of State machine

# 4. History

| Date | Author | Modified part | Description |
| --- | --- | --- | --- |
| 2017/03/28 |  | All | New creation |